HEALTH AND SAFETY DATA SHEET

CONVOCLEAN FORTE

### Product Identifier:  ConvoClean Forte, cleaner

Use of the substance:  Washing and cleaning products (including solvent based products)

### Hazards identification

### Classification of the substance or mixture

### Classification according to Directive 67/548/EEC or 1999/45/EC

### Indications of danger: Corrosive, R-phrases: Causes burns

Classification according to Regulation (EC) No. 1272/2008 [CLP]

Hazard categories:  Substance or mixture corrosive to metals: Met. Corr. 1, Skin corrosion/irritation: Skin Corr. 1B, Serious eye damage/eye irritation: Eye Dam. 1, Hazard Statements:  May be corrosive to metals.  Causes severe skin burns and eye damage.

### Label elements

Hazardous components which must be listed on the label: caustic potash, potassium hydroxide

Signal word: Danger

Pictograms: GHS05

### Hazard statements

H290 – May be corrosive to metals.

H314 – causes severe skin burns and eye damage.

Precautionary statements

P260 – Do not breathe dust/fume/gas/mist/vapours/spray.

P280 – Wear protective gloves/protective clothing/eye protection/face protection.

P301+P330+P331 – IF SWALLOWED:  Rinse mouth.  Do NOT induce vomiting.

P303+P361+P353 – IF ON SKIN (or hair):  take off immediately all contaminated clothing.  Rinse skin with water/shower.

P305+P351+P338 – IF IN EYES: Rinse cautiously with water for several minutes.  Remove contact lenses, if present and easy to do.  Continue rinsing.

P310 – Immediately call a POISON CENTRE/doctor.

Other Hazards

The preparation is dangerous in the sense of Directive 1999/45/EC

This preparation is hazardous in the sense of regulation (EC) No 1272/2008 [GHS].

### Composition/information on ingredients

Hazardous components

Potassium Hydroxide, Caustic Potash ; EC-No : 215-181-3; Percentage <5%

CAS-No. : 1310-58-3 Classification according to Directive 67/548/EEC - C – Corrosive, Xn -  Harmful R22-35 Index No. 019-002-00-8 , Classification according to Regulation (EC) No: 1272/2008 [CLP]: Acute Tox. 4, Skin Corr. 1A; H302 H314

2-(2-BUTOXYETHOXY)ETHANOL; EC-No. : 203-961-6; Percentage <5%

CAS-No. : 112-34-5 Classification according to Directive 67/548/EEC: X - Irritant; R36

Index No. 603-096-00-8, Classification according to Regulation (EC) No. 1272/2008 [CLP]: Eye Irrit. 2; H319

Labelling for contents according to Regulation (EC) No. 648/2004: <5% phosphates, <5% non-ionic surfactants, <5% amphoteric surfactants.

#### First-aid measures

General

Remove contaminated, saturated clothing immediately. First aider: Pay attention to self-protection! Provide fresh air

After inhalation

In case of inhaling spray mist, consult a doctor immediately and show him packing or label.

After skin contact

After contact with skin, wash immediately with: water. Subsequently wash again with: Polyethylene glycol 400. Do not wash off with soap or other alkaline cleaning agents. In case of skin irritation, seek medical treatment.

After eye contact

In case of contact with eyes, rinse immediately with plenty of flowing water for 10 to 15 minutes holding eyelids apart. Subsequently consult an ophthalmologist. Protect uninjured eye

After ingestion

Rinse mouth thoroughly with water. Let water be swallowed in little sips (dilution effect). Do NOT induce vomiting. Call a physician immediately.

Most important symptoms and effects, both acute and delayed

Causes burns. If swallowed danger of perforation of the oesophagus and the stomach  (strong corrosive effects).  Caution if victim vomits:  Risk of aspiration!  Eye Dam. 1.

Indication of any immediate medical attention and special treatment needed

First Aid, decontamination, treatment of symptoms.

## Fire-fighting measures

Suitable extinguishing media

Use water spray jet to protect personnel and to cool endangered containers.

Co-ordinate fire-fighting measures to the fire surroundings. Water. Foam. Dry extinguishing powder. ABC powder. Carbon dioxide (CO2). Water spray.

Special hazards arising from the substance or mixture

In case of fire may be liberated:  Carbon monoxide.  Nitrogen oxides (Nox). Carbon dioxide (CO2).

Advice for firefighters

The product itself does not burn.  In case of fire:  Wear self-contained breathing apparatus.

Additional information

Contaminated fire-fighting water must be collected separately. In case of fire and/or explosion do not breathe fumes.

## Accidental Release Measures

Personal precautions, protective equipment and emergency procedures

Avoid contact with skin and eyes. Provide adequate ventilation. Wear personal protection equipment. High slip hazard because of leaking or slipped product.

Environmental precautions

Do not allow to enter into surface water or drains.  Do not allow to enter into soil/subsoil.

Methods and material for containment and cleaning up

Absorb with liquid-binding material (e.g. sand, diatomaceous earth, acid –or universal binding agents). Treat the recovered material as prescribed in the section on waste disposal. Clear contaminated areas thoroughly. Dilute with plenty of water. The contaminated area should be cleaned up immediately with water.

Reference to other sections

See protective measures under Handling/Storage, Exposure Controls/Personal Protection and Disposal Considerations.

Handling and storage

Precautions for safe handling

Advice on safe handling

If handled uncovered, arrangements with local exhaust ventilation should be used if possible.  If local exhaust ventilation is not possible or not sufficient, the entire working area should be ventilated by technical means.. Conditions to avoid: Formation of aerosol. All work processes must always be designed so that the following is as low as possible: inhalation. skin contact.

Advice on protection against fire and explosion

The product itself does not burn

Conditions for safe storage including any incompatibilities

Requirements for storage rooms and vessels

Suitable floor material: Leachate-proof. Handle and open container with care. Keep container tightly closed. Suitable material for container: polyethylene. Unsuitable material for container: metal.

Advice on storage compatibility

Do not store together with: Acid. Keep away from food, drink and animal feeding stuffs.

Further Information about storage conditions

Do not store at temperatures under: 5 degrees Celsius. Protect against direct sunlight. Keep only in the original container in a cool, well-ventilated place

Specific end use:  Observe instructions for use.

###### Exposure Controls/Personal Protection

### Control parameters – Exposure limits (EH40)

### 2-(2-BUTOXYETHOXY)ETHANOL; CAS-No: 112-34-5 10ppm, 67.5mg/m3 , TWA (8h), WEL, 15ppm, 101.2mg/m3  STEL(15 min) WEL,

### POTASSIUM HYDROXIDE CAS No. 1310-58-3 TWA(8h) WEL 2mg/m3 STEL(15 min) WEL

### Exposure controls

### Refer to Handling and Storage. Additional information about structure of technical plants: Work in well ventilated zones or use proper respiratory protection. Do not breathe gas/fumes/vapour/spray

### Protective and hygiene measures

### Avoid contact with skin and eyes. Remove contaminated, saturated clothing immediately. Wash hands before breaks and at the end of work. When using do not eat, drink or smoke. Protect skin by using skin protective cream.

### Respiratory protection

### Respiratory protection required in case at: insufficient ventilation, aerosol or mist generation. Suitable respiratory protective equipment: particulates filter device (DIN EN 143). P2

Hand protection

Tested protective gloves are to be worn: Suitable material: Butyl rubber. (0.5mm), NBR (Nitrile rubber)(0.35mm), penetration time (maximum wearing period: >480 min

Recommended protective gloves brand: Camatril Velour, Manufacturer: KCL or comparable articles from other companies.

Eye/face protection

Tightly sealed safety glasses.

Skin protection

Suitable protection of body: protective apron.

###### Physical and chemical properties

### Form :		                Liquid.

### Colour :		                Red.

### Odour :		                Characteristic.

Boiling point / range :	  98C

Flash point :	  not applicable

Density  (at 20C )             	  1.1g/cm3

Solubility in water : (at 20C)    Miscible

pH value (at 20C )	   14

Solubility in other solvents: Ethanol, Acetone

Vapour pressure (at 20°C)         23 hPa

Vapour pressure (at 50°C)         123hPa

Other information: Corrosion rate on steel or aluminium surfaces is >=55mm per year at a test temperature of 6,25C

###### Stability and reactivity

Reactivity   Reacts with acid and carbon dioxide.

Chemical stability:  The product is stable under storage at normal ambient temperatures.

Possibility of hazardous reactions

Exothermic reactions with acid.  Violent reaction with acid.  Possibly extensive generation of hydrogen on contact with amphoteric metals (e.g. aluminium, lead, zinc) (explosive hazard!)

Conditions to Avoid   Protect from direct sunlight

Incompatible Materials:  Light metals. Acid. Oxidizing agent, strong. Peroxides. Zinc. Copper. Copper containing metals

Hazardous decomposition products:  Carbon dioxide

Further information

Corrosive to metals. Slowly corrodes aluminium and zinc under hydrogen evolution.

###### Toxicological information

Acute Toxicity

Caustic Potash, Potassium Hydroxide  CAS No. 1310-58-3  Exposure Route – Oral

LD50   273mg/kg   Rat   RTECS

Irritation and Corrosivity  Causes severe skin burns and eye damage.

Ecological Information

Toxicity

Caustic Potash, Potassium Hydroxide   CAS No. 1310-58-3 Acute fish toxicity LC50 80mg/l    96h    Gambusia affinis    IUCLID

Persistence and degradability

The product is an alkali.  Before discharge into sewage plants the product normally needs to be neutralized. The surfactants contained in the product are biodegradable according to the requirements of the Detergent Directive 648/2004/EC.

Bioaccumulative potential

No indication of bio-accumulation potential

Mobility in soil

If product enters soil, it will be mobile and may contaminate groundwater.

Results of PBT and vPvB assessment

The components in this formulation do not meet the criteria for classification as PBT or vPvB

Disposal considerations

Waste Treatment Methods

Advice on disposal

Waste disposal according to applicable legislation.

Waste disposal number of waste from residues/unused products

060204 WASTES FROM INORGANIC CHEMICAL PROCESSES; wastes from the MFSU of bases; sodium and potassium hydroxide. Classified as hazardous waste.

Waste disposal number of used product

060204 WASTES FROM INORGANIC CHEMICAL PROCESSES; wastes from the MFSU of bases; sodium and potassium hydroxide. Classified as hazardous waste.

Waste disposal number of contaminated packaging

150110 WASTE PACKAGING; ABSORBENTS, WIPING CLOTHS, FILTER MATERIALS AND PROTECTIVE CLOTHING NOT OTHERWISE SPECIFIED; packaging (including separately collected municipal packaging waste); packaging containing residues of or contaminated by dangerous substances. Classified as hazardous waste.

Contaminated packaging

Water. Contaminated packing must be completely emptied and can be re-used following appropriate cleaning.  Cleaned containers may be recycled.

### Transport information

### Land transport  (ADR/RID)

###### Class :	8

### UN number :	1814

Packing Group:           II

Hazard Label:               8

### Classification Code :   C5

Limited Quantity:       1L

Hazard No:                   80

### Tunnel restriction code : E

### Proper shipping name: Potassium Hydroxide Solution

Other Applicable Information: E2 Transport Category 2

### Inland Waterways Transport (ADN)

### UN number:            1814

Packing group :	     II

Transport Hazard Class  8

### Hazard Label :	       8

### Classification Code:   C5

### Limited Quantity:      1L

Proper Shipping Name: Potassium Hydroxide Solution

Other Applicable Information: E2

### Maritime transport (IMDG)

### Transport Hazard Class:8

### EmS number :          F-A / S-B

### UN number :	1814

### Proper shipping name:    Potassium hydroxide solution

### Packing group :	II

### Hazard Label :	8

Limited Quantity: 1L

Special Provisions: E2

### Air transport (ICAO)

### Transport Hazard Class : 8

### UN number :	1814

### Proper shipping name: Potassium Hydroxide Solution

### Packaging group :	II

### Label :		8

Special Provisions: A3 A803

Limited Quantity Passenger: 0.5L

IATA packing Instruction s- Passenger: 851

IATA Max Quantity – Passenger: 1L

IATA Packing Instructions- Cargo: 855

IATA Max Quantity 0 Cargo: 30L

Other Applicable Information; E2, Y809, Y840

### Environmental hazards: no

Special precautions for user: Call a POISON CENTRE/doctor if  you feel unwell.  Warning Corrosive. May be corrosive to metals

###### Regulatory information

Additional Information

The surfactants contained in the product are biodegradable according to the requirements of the Detergent Directive 648/2004/EC.  Data to support this assertion are held at the disposal of the competent authorities of the Member States and will be made available to them, at their direct request or at the request of a detergent manufacturer.

### National regulatory information

### Employment restrictions: Observe employment restrictions for young people

### Water contaminating class (D): 1- slightly water contaminating

### Chemical Safety assessment: Chemical safety assessments for substances in this mixture were not carried out

Other information

Full text of R-phrases referred to under section Hazard Identification and Information on Ingredients:

22: Harmful if swallowed

35: Causes severe burns

36: Irritating to eyes

Full text of H- and EUH phrases referred to under section Hazard Identification and Information on Ingredients

H290:  May be corrosive to metals.

H302: Harmful if swallowed

H314: Causes severe skin burns and eye damage

H319: causes serious eye irritation

Further information

The product should only be handled by persons over the age of 18, who were informed sufficiently about the dangerous nature of the product and about the necessary safety precautions.  The above information describes exclusively the safety requirements of the product and is based on our present-day knowledge. The information is intended to give you advice about the safe handling of the product named in the safety data sheet, for storage, processing, transport and disposal. The information cannot be transferred to other products. In the case of mixing the product with other products or in the case of processing, the information on this safety data sheet is not necessarily valid for the new made-up material.

Date: 10th February 2015 Revision No: 2,00